Testing GPS Doppler Devices for Speedsurfing: Are They All Equally Good?

If you have a headache you can take a paracetamol or an ibuprofen. They work in a different way, but give the same result. That is also the conclusion after testing the GW60, Coros ApexPro/Vertix1, Coros Vertix2 and the Motion LCD and Motion Mini.

In practice, we see that the 10-second measurements show very few differences. The differences in measurements over 2 seconds are slightly larger.

GPS devices use several techniques to arrive at the speed measurements, and differ from each other with this:

- Difference in Kalman filters (error correction)

- Difference in measurement points per second (1Hz, 5Hz, 10Hz)

- Difference in GPS only or using multiple GLNSS systems and/or Dual Band

The Kalman filter is a calculation method that continuously corrects the measured speeds, to a greater or lesser extent, based on the measurements before and after the measurements. As a simple example: measurement point A = 50 km/h, point B = 60 km/h, Point C = 55 km/h. The Kalman filter will then correct point B to 53 km/h. After all, an acceleration of 10 km/h within 1 second and then a 5 km/h deceleration does not seem real. The Kalman filter says that the probability that Point B is a false reflection of reality is too great and will then correct it. All GPS devices correct in this way on almost all data points. Only in a very small number of cases does the Kalman filter fully agree with the measured speed. Each supplier determines to what extent and when the Kalman filter should intervene.

The filter in the GW60 (5Hz), which only calculates with GPS and in practice with 8 to 9 satellites, proves that a Kalman filter can do its job well. Technically speaking, this is the least advanced GPS device, but in practice it does not show significantly more deviations. Apparently you can do a reliable measurement with 8 to 9 GPS satellites at 5Hz.

If we then look at the measuring points per second, with 1Hz there seems to be a greater chance of deviations from measuring point to measuring point, compared to a 5Hz or 10Hz measurement. In other words: to calculate a 2 seconds maximum speed average with 2 measuring points (1Hz), 10 measuring points (5Hz) or 20 measuring points (10Hz) makes a difference in precision. The Coros GPS devices measure with 1Hz.

How is it possible that the Coros GPS devices still end up at (almost) the same speeds?

This is compensated for with the Coros ApexPro and Vertix 1 by using 3 satellite systems simultaneously (GPS/GLONASS or Beidou and Galileo) and by calculating with an average of 12 to 13 satellites.

The Kalman filter in the ApexPro and Vertix1 at the time of writing (FIRMWARE V2.61.0) is set too rigidly and rounds the measured values ​​down more than the other GPS devices. In practice, this sometimes results in lower speeds of 1.9 km/h every 2 seconds. On average, the measured speed is 0.45 km/h lower than the other devices in 2 seconds. These differences level out more at 10 seconds, but measured over more than 50 runs of 10 seconds, it is still 0.1 km/h lower than the other devices. The maximum measured deviation in 10 seconds compared to the other devices is -0.56 km/h.

The Coros Vertix2 focuses on the use of all satellite systems simultaneously and also does so in Dual Band. The second band measurement verifies the measurement on the first band. We have found that Dual Band provides a significant improvement in non-ideal measurement conditions. With “all systems on” the Vertix2 sees between 30 and 40 satellites and selects the 12 satellites with the best signal-to-noise ratio (HDOP 0.4 [Horizontal Dilution of Precision]). Fewer pulses per second therefore do not lead to a less reliable end result due to this technique.

The Motions use 2 or 3 satellite systems. At 10Hz with 2 (GPS+Glonass) and at 5Hz Galileo is added. The Motion calculates with a maximum of 18 satellites simultaneously. The advantage of those extra satellites comes at the expense of the average signal-to-noise ratio. We see an increasing HDOP value (the higher the HDOP value is, the higher the chance of measurement errors).

In practice we see that the Motion regularly only uses 12 to 14 satellites. The HDOP values ​​remain virtually the same. With the 5Hz setting (with Galileo) it often takes the maximum number of satellites with it.

We see that the Motion at 5Hz shows slightly less outliers on the individual data points than at 10Hz (we expect this may be due to Galileo). At the end of the ride you will not see those differences and 5Hz or 10Hz is equivalent.

Which one should you buy now?

If you already own one of these devices, there is no need to switch.

They all perform equally at 5x10 seconds and that is what GPS Speedsurfing is all about.

With the exception of the ApexPro and Vertix1, which display slightly lower speeds.

Statistical:
-	GW-60 has a lower 10 seconds than the MOTION and the VERTIX 2 (all dual), 49.803
 	vs. 49,847 and 49,895, respectively

- 	Both Apex pros (CAP-GL-GAL and CAP-BEI-GAL) have a lower 10 seconds than the MOTION and

the Vertix 2 (all dual), 49,732 and 49,734 vs. 49,847 and 49,895, respectively

- 	The MOTION and the Vertix 2 (all dual) do not differ significantly from each other in the

max 10 seconds.

Advantages and disadvantages per device:

- 	The limitation of the GW60 is in limited satellite reception, but it advantage is the 5Hz

measurement. The disadvantage is the limited battery capacity and the vulnerability

of the strap and case

- 	The limitation of the ApexPro/Vertix1 is in the 1Hz measurement, but the advantage is it

use of 3 satellite systems. In addition, Coros is very user-friendly.

- 	The limitation of the Vertix2 is in the 1Hz measurement, the advantage is the use of all

5 satellite systems simultaneously in Dual Band. In addition, Coros is very user-friendly.

- 	The advantage of Motion LCD is the measurement in 5Hz (3 satellite systems) or 10Hz

(2 satellite systems). The disadvantage is the cumbersome way to read the data.

- 	The advantage of Mini Motion is the measurement in 5Hz (3 satellite systems) or 10Hz

(2 satellite systems). The disadvantage is the cumbersome way to read the data.

Since the Motion Mini does not have a display, you will not be able to see results on

the water. You still need another GPS watch for that. On the other hand, this is the

most economical device.

Future:

We can safely say that the current devices are reliable. Certainly in the right circumstances. When the measurement conditions are more difficult, there is still a (small) gain to be made.

The ideal handheld device communicates with all satellite systems simultaneously in Dual Band in 5Hz or 10Hz. Unfortunately it is not for sale at the moment!

Additional information about Dual Band and DOP values:

Is dual-band a GPS superpower?. New dual-band GPS/GNSS technology… | by Mike Horton | Medium

What is DOP, PDOP, HDOP, VDOP and where can I find this information? - marXact Knowledge Base